| COUNTY OF FRESNO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ADDENDUM NUMBER: THREE |  |  |  |  |  |  |  |  |  |
|  |  |  |  | RFQ NUMBER: |  |  |  |  |  |
| August 8, 2007 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | PURCHASING USE | /tmp/in/input.doc |  |  |  |
|  |  |  |  |  | [typist's initials] |  |  |  |  |
|  | IMPORTANT: SUBMIT QUOTATION IN SEALED PACKAGE WITH QUOTATION NUMBER, CLOSING DATE AND BUYER’S NAME MARKED CLEARLY ON THE OUTSIDE TO: |  |  |  |  |  |  |  |  |
|  | COUNTY OF FRESNO, Purchasing 4525 EAST HAMILTON AVENUE FRESNO, CA 93702-4599 |  |  |  |  |  |  |  |  |
|  | Closing date of bid will be at 2:00 p.m., on August 23, 2007. QUOTES WILL BE CONSIDERED LATE WHEN THE OFFICIAL PURCHASING TIME CLOCK READS 2:00 P.M. |  |  |  |  |  |  |  |  |
|  | Quotes will be opened and publicly read at that time. All quotation information will be available for review after contract award. |  |  |  |  |  |  |  |  |
|  | Clarification of specifications are to be directed to: Craig Nickel, phone (559) 456-7110, FAX (559) 456-7831. |  |  |  |  |  |  |  |  |
|  |  | NOTE THE following and attached ADDITIONS, DELETIONS AND/OR CHANGES TO THE REQUIREMENTS OF REQUEST FOR PROPOSAL NUMBER: AND INCLUDE THEM IN YOUR RESPONSE. PLEASE SIGN AND RETURN THIS ADDENDUM WITH YOUR PROPOSAL. - Delete pages 13 through 21. - Replace pages 13 through 21 with the following pages (revised pages 13 through 21). - The trucks will be used to pull bottom dump trailers. Good ground clearance is necessary, as the pictures that are included will demonstrate. |  |  |  |  |  |  |  |

| ACKNOWLEDGMENT OF ADDENDUM NUMBER THREE TO RFQ |  |  |
| --- | --- | --- |
| COMPANY NAME: |  |  |
| (PRINT) |  |  |
| SIGNATURE: |  |  |
| NAME & TITLE: |  |  |
| (PRINT) |  |  |

tRUCKS (tRACTORS)

The desired truck(s) shall be equal to a Freightliner, International, Kenworth, Peterbilt, etc. with specifications as follows:

SPECIFICATIONS

|  | Comply Does Not Comply |
| --- | --- |
| ENGINE: |  |
| Cat C-13 or equal |  |
| Horsepower: 430 hp @ 2100 rpm Minimum |  |
| Torque: 1650 LB./FT. @ 1200 RPM |  |
| Meets or exceeds all current and proposed CARB/EPA Emissions Standards and Certifications |  |
| Jacobs Engine Brake |  |
| Magnetic Oil Drain plug |  |
| Bendix 16.1 CFM Air compressor |  |
| Alternator: 160 AMP |  |
| Batteries: Group 31 threaded stud, 1900 CCA - Qty 3 |  |
| Battery box, mounted under driver’s side door, or behind fuel tank |  |
| Electrical engine integral shutdown system |  |
| Stationary cab mounted single vertical exhaust or frame mounted. Dual Exhaust is not acceptable |  |
| Vertical curved tail pipes |  |
| Stainless steel muffler/tailpipe shields |  |
| Horton On/off fan drive – Air operated |  |
| Oil Filter: Full flow |  |
| Radiator: 1600 sq.-in (approximate) |  |
| Extended Life Antifreeze: -34 degrees (approximate) ethylene glycol |  |
| Coolant Hoses: Gates Blue Stripe or equal |  |
| Constant tension hose clamps |  |
| Flywheel Housing: Aluminum |  |
| Starter: 12-Volt heavy-duty |  |
| Factory Air Conditioning |  |
| Power Steering: 1.5 quart frame or engine mounted |  |
| TRANSMISSION |  |
| Eaton Fuller RTLO 16913 NO EXCEPTION |  |
| 13 – Speed with low gearing (low first drive gear) |  |
| County will test drive a comparable vendor’s truck for |  |
| low end suitability and desired gear ratios |  |
| Clutch 15.5 inch. 7 spring, ceramic |  |
| Manual adjust with grease bearing |  |
| Remote Lube Hoses for clutch release-bearing (if available) |  |
| Driveline: Heavy Duty, minimum 4.5” x .180 wall tubing |  |
| Clutch Housing: Aluminum |  |
| Transmission Cooler |  |
| FRONT SUSPENSION |  |
| Cab Forward Front Axle Approximately 92” to BOC |  |
| Capacity: 14,600 LB |  |
| Taperleaf Springs with shocks: 14,600 LB |  |
| Meritor: FG-941 front axle or equal |  |
| 16.5 X 5 front brakes (or equal), no Centrifuse |  |
| Non asbestos brake linings |  |
| Brake dust shields |  |
| Front Shock Absorbers |  |
| Oil Seals: Chicago Rawhide Scotseal Plus XL or equal. |  |
| Spindle Nuts: Standard |  |
| Front Hubcaps: Vented, oil |  |
| Automatic Slack Adjusters: Meritor or equal |  |
| Power Steering Reservoir: Sight glass or dip stick |  |
| Rubber Bushings: Maintenance Free |  |
| REAR AXLE |  |
| Capacity: 23,000 LB |  |
| Axle: Single, Meritor RS23-161 R-SRS or equal |  |
| Axle Ratio: 3.91 |  |
| Rear Axle Carrier Housing: Iron |  |
| Differential Traction: Driver Controlled – Air Locker |  |
| Rear Brakes: Meritor 16.5 x 7 Q+ cam rear brakes |  |
| Double anchor points |  |
| Brake Linings: Non-asbestos |  |
| Brake Drums: Cast iron |  |
| Oil Seals: Chicago Rawhide Scotseal (or equal) |  |
| Slack Adjusters: automatic |  |
| Airliner 23,000 lb. Suspension (or equal) |  |
| Airliner high position ride height (or equal) |  |
| Single rear suspension auto leveling valve |  |
| Rear shock absorbers |  |
| Manual Dump Valve: Air Suspension, with indicator light, gauge, and buzzer |  |
| BRAKE SYSTEM EQUIPMENT |  |
| Anti-lock (ABS) |  |
| Reinforced nylon, fabric braid chassis air lines (or equal) |  |
| Brake Valves: Standard |  |
| Air Dryer; Bendix AD / IS style or equal |  |
| Reservoirs: Steel |  |
| Air Tanks: Manual Pull Cables |  |
| TRAILER CONNECTIONS |  |
| Trailer Air Hose: 12 foot coiled |  |
| Slide Bar: Cab Mounted Pogo stick |  |
| Primary connector receptacle center pin powered through ignition |  |
| Trailer Cable Receptacle: SAE J560 7-way |  |
| 12 foot detachable coiled primary electrical cable with SAE J560 |  |
| Wiring for train of bottom dump trailers |  |
| WHEELBASE |  |
| 170 inch wheelbase |  |
| FRAME |  |
| Frame Rail: 10.5/8 x 5/16 inches steel rails (approximate) |  |
| 5/16 inch rail thickness |  |
| Crossmember: Aluminum or steel |  |
| CHASSIS EQUIPMENT |  |
| Deck Plate flush mounted between frame rails covering entire area with appropriate grab handles and steps |  |
| Front Bumper: Aluminum front bumper or chrome plated steel |  |
| Center tow hooks |  |
| Rear mounted mud flaps |  |
| Front wheel Mud flaps |  |
| Rear fenders: Aluminum, 60-inch |  |
| Triangle Reflector Kit |  |
| FIFTH WHEEL AND EQUIPMENT |  |
| Holland fixed fifth wheel, FW8-7883XR or equal |  |
| Fifth Wheel Height: 8.62 inches |  |
| Fifth wheel setting: 6 inches in front C/L bogie |  |
| Unladen Height: 50 inches (minimum) |  |
| Unladen Height: 52 inches (maximum) |  |
| FUEL TANK AND EQUIPMENT |  |
| Fuel Tanks: 1) 70 gallon, driver’s side |  |
| Aluminum construction, 19 inches minimum |  |
| Mounted under cab or behind doors or cab |  |
| 1) 70 gallon, passenger side |  |
| Aluminum construction, 23 inches |  |
| Mounted under cab or behind cab |  |
| Fuel and Water Separator |  |
| BATTERY BOX AND TOOL BOX |  |
| Battery Box: with non-slip deck behind fuel tanks |  |
| Rubber battery pad in bottom of box |  |
| Aluminum toolbox, with non-slip, behind fuel tank on side of cab. Dimensions: 12 inch deep x 16 inch height x 32 inch length |  |
| FRONT TIRES, WHEELS AND HUBS |  |
| Tires: equal to Michelin XZA2 |  |
| Size: 11R 24.5 14 Ply |  |
| Pre-set bearing front hubs |  |
| Wheels: Aluminum |  |
| Accuride 29644A 24.5x 8.25 10 hub pilot or equal |  |
| REAR DRIVE TIRES, HUBS AND WHEELS |  |
| Tires: equal to Michelin XDA H/T |  |
| Size: 11R 24.5 14 Ply |  |
| Aluminum Rear Hubs |  |
| Wheels: Aluminum 24.5 x 8.25 or equal |  |
| CAB EXTERIOR |  |
| Cab air – rear mount suspension |  |
| Hood: Aluminum, composite or fiberglass |  |
| Fender rock liners (if aluminum) |  |
| View window – Optional item, see Quotation page – window in passenger door |  |
| Windows air or electric operated |  |
| Door lock and ignition keyed alike |  |
| Outside sun visor: stainless steel or composite |  |
| Conventional Cab: 120 inches (approximate) |  |
| Radiator bug screen |  |
| Grab handles: left and right hand sides |  |
| Horn: single air horn |  |
| Marker lights |  |
| Fog lights |  |
| Mirror mounted turn signal lights |  |
| Mirrors dual stainless steel heated |  |
| Width: 102 inches |  |
| Convex mirrors: left and right hand sides |  |
| Convex mirrors over passenger door NO EXCEPTION |  |
| Reflectors: Standard, side and rear |  |
| Rear reflective device |  |
| Tinted rear window |  |
| Tinted door windows |  |
| Tinted windshield |  |
| Windshield washer reservoir |  |
| CAB INTERIOR |  |
| Dome light |  |
| Vinyl interior |  |
| Door Trim |  |
| Floor Mats with insulation |  |
| Roof Mounted Console |  |
| Cup Holder |  |
| Fire Extinguisher: 5 pound capacity |  |
| Heater, Defroster and Air Conditioning |  |
| HVAC controls with re-circulation switch |  |
| Fresh Air Intake Filter(s) |  |
| Cab insulation or silencer upgrade package |  |
| Circuit Breakers: Automatic or manual |  |
| Turn Signal Switch: Self canceling |  |
| Driver Seat: Bostrom Air Ride 915 high back or equal |  |
| Passenger Seat: Bostrom Fixed 915 high back or equal |  |
| Seat Retractors: 3-point |  |
| Steering Column: adjustable |  |
| Sun Visors: drivers and passenger |  |
| INSTRUMENT PANEL AND CONTROLS |  |
| Low air pressure light and buzzer |  |
| Single brake application air gauge |  |
| Primary and secondary air pressure gauges |  |
| Cruise Control |  |
| Diagnostic Interface Connector, SAE J1587/1708 |  |
| Fuel Gauge: Electric |  |
| Engine Coolant Gauge: Electric |  |
| Oil Temperature Gauge: Electric |  |
| Engine Oil Pressure Gauge: Mechanical |  |
| Tachometer: 0 - 3,000-RPM Range |  |
| Speedometer: Electrical with Odometer |  |
| Engine Fan Override Switch |  |
| Hour Meter: Hobbs Part Number 85097-02 NO EXCEPTION (County Standard) |  |
| Voltmeter |  |
| Radio: AM/FM |  |
| Power Point – 12 volt |  |
| Cigar Lighter-12 Volt |  |
| Tractor Protection Valve |  |
| Windshield Wiper Motor with delay |  |
| CB radio installed with antenna |  |
| Cigar Lighter-12 volt |  |
| PAINT |  |
| Cab color equal to Sherwin Williams GCA-2030-00 (white) |  |
| Chassis: Black, with solid Polyurethane Paint |  |
| ADDITIONAL ITEMS |  |
| Pre-wired for County Radio – County will furnish and install County owned radio upon delivery and acceptance of these trucks |  |
| Back up alarm |  |
| Dump switches: Factory installed with guards |  |
| Work lights: Flush or cab mounted Rear Facing – Quantity 2 (minimum) |  |