| COUNTY OF FRESNO |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ADDENDUM NUMBER: TWO (2) |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | RFQ NUMBER: 578-4066 |  |  |  |  |  |  |  |  |
| March 24, 2005 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | PURCHASING USE |  |  |  |  |  |
|  |  |  |  |  |  |  | JLG | /tmp/in/input.doc |  |  |  |
|  | IMPORTANT: SUBMIT QUOTATION IN SEALED PACKAGE WITH QUOTATION NUMBER, CLOSING DATE AND BUYER’S NAME MARKED CLEARLY ON THE OUTSIDE TO: |  |  |  |  |  |  |  |  |  |  |
|  | COUNTY OF FRESNO, Purchasing 4525 EAST HAMILTON AVENUE FRESNO, CA 93702-4599 |  |  |  |  |  |  |  |  |  |  |
|  | Closing date of bid will be at 2:00 p.m., on APRIL 14, 2005. QUOTES WILL BE CONSIDERED LATE WHEN THE OFFICIAL PURCHASING TIME CLOCK READS 2:00 P.M. |  |  |  |  |  |  |  |  |  |  |
|  | Quotes will be opened and publicly read at that time. All quotation information will be available for review after contract award. |  |  |  |  |  |  |  |  |  |  |
|  | Clarification of specifications are to be directed to: KEN VOZZA, phone (559) 456-7110, FAX (559) 456-7831. |  |  |  |  |  |  |  |  |  |  |
|  | NOTE THE FOLLOWING AND ATTACHED ADDITIONS, DELETIONS AND/or CHANGES TO THE REQUIREMENTS OF REQUEST FOR QUOTATION NUMBER: 578-4066 AND INCLUDE THEM IN YOUR RESPONSE. PLEASE SIGN AND RETURN THIS ADDENDUM WITH YOUR QUOTATION. - THE CLOSING DATE FOR RFQ NO. 578-4066 HAS BEEN CHANGED TO APRIL 14, 2005 AT 2:00 P.M. |  |  |  |  |  |  |  |  |  |  |
|  | ACKNOWLEDGMENT OF ADDENDUM NUMBER TWO (2) TO RFQ 578-4066 |  |  |  |  |  |  |  |  |  |  |
|  | COMPANY NAME: |  |  |  |  |  |  |  |  |  |  |
|  | (PRINT) |  |  |  |  |  |  |  |  |  |  |
|  | SIGNATURE: |  |  |  |  |  |  |  |  |  |  |
|  | NAME & TITLE: |  |  |  |  |  |  |  |  |  |  |
|  | (PRINT) |  |  |  |  |  |  |  |  |  |  |

- The “SPECIFICATIONS/REQUIREMENTS” section of RFQ No. 578-4066 identifies various equipment design characteristics (features). Those design characteristics are a means of attaining specific performance objectives. Alternate equipment designs that yield equal performance may be acceptable. County will determine acceptability.

Vendors may offer alternative equipment designs. However, the equipment offered must perform as required by the County. Vendors offering alternative designs must disclose how that design differs from the design identified by the County. Additionally, the vendor must provide a detailed explanation of how the alternate design will satisfy the County’s performance requirements.

The following section of this Addendum lists various items found under the “SPECIFICATIONS/REQUIREMENTS” section of RFQ No. 578-4066. This list also includes the performance objective for each item. The equipment offered under this RFQ must meet the County’s performance requirements.

NOTE: The above conditions and requirements are applicable for all items stated in the original RFQ document under the “SPECIFICATIONS/REQUIREMENTS” section, even if not included in the list below.

  - ENGINE

Engine enclosure, This serves a two-fold purpose not only does it provide for noise reduction when using the equipment in residential areas but also reduces exposure to the elements, especially the sun, when equipment is stored outdoors, lessening the deterioration of components like hoses, wires and belts. Also in this configuration the radiator is totally enclosed providing for additional protection from damage.

  - INTEGRATED AIR COMPRESSOR

Compressor and hose reel enclosed, this again allows for noise reduction by enclosing the compressor and extended component life (hoses, belts) by limiting exposure to weather.

  - TRAILER/FRAME

A rectangular tube frame and cross members is easer to maintain and keep clean do to the fact that there are not flat hidden areas that can trap and hold various types of road debris especially those that may be associated with the crack filling operation.

Flat reinforced fenders not only allow for an area to place material blocks if for some reason they cannot be immediately loaded into the melter but also enclose the backside of the wheels preventing anything from inadvertently coming in contact with the backside of the tires.

  - MATERIAL TANK

Insulated steel doors, strength and heat retention.

Loading doors on both sides, convenience and safety of being able to load on either side. No need to shuffle material to one side of trailer lessening chance of injury such as a trip and fall by stepping over tongue or walking through work area around various hoses such as product and air. Ability to load on either side staying clear of traffic at all times.

Loading height less then 49", the lower the loading height the less likelihood of back injury and we need to do all that is possible in this area.

Door opening 15"x26", makes the melter easer to load results in increased efficiency and reduced chance of injury.

  - BURNER

Shroud to protect the burner, a hinged door is considered necessary for access.

  - PUMPING SYSTEM

External mounted pump, this is for serviceability if for any reason there in a pump malfunction the tank must be emptied before pump can be repaired. This would require the draining of the tank of hot material though a gravity-fed drain into some type of container for later remelting. Anytime hot material is handled in this way there is a chance of injury.

30 GPM pump allows for an abundant supply of material to facilitate the use of two hoses at the same time as well as the constant circulation of material through the system keeping all components at a consistent temperature.

Material plumbing and valves in heated cabinet, keeping these items in an area protected from the elements not only is beneficial during the time of operation but also during times of storage.

  - HEATED HOSES

Minimum of 20' hose, the longer hose, the larger area that can be covered without relocating the melter trailer. This increases efficiency because the less often that an operator is needed to move the trailer the more they can be involved in other areas of the crack filling operation.

62" boom height, keeps boom higher above work area lessening the likelihood of inadvertently bumping into it therefore reducing the chance of injury to shoulder or back. This also supports one end of the hose for a longer distance away from the trailer.

  - SEALING WAND

The ability to disassemble the wand in the middle is important since it allows easier maintenance plus individual portions can be replaced instead of a full component resulting in a cost savings.

- Refer to the SPECIFICATIONS/REQUIREMENTS section of RFQ No. 578-4066.

Reference “Material Tank” (Page No. 18).

The requirement stating “No auxiliary electric heat elements (no exceptions)” shall be deleted in its entirety.  The following requirement shall be added to this section:

“The unit must provide a self contained heating source and shall not require an external source to heat material, including power.”

- All items listed under the SPECIFICATIONS/REQUIREMENTS section of the RFQ may be satisfied by a design other than that specified. However, any such alternative design must produce the intended result.  The bidder may offer an alternate for any specification that appears to limit the equipment to a specific design type. Terms such as “only”, “will”, “will not be considered”, “must” etc. may be disregarded.

The offer of alternative design features must be supported in accordance with the instructions stated under No. One, above.