| COUNTY OF FRESNO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ADDENDUM NUMBER: TWO |  |  |  |  |  |  |  |  |  |
|  |  |  | RFQ NUMBER: |  |  |  |  |  |  |
| May 20, 2011 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | PURCHASING USE |  |  |  |  |  |
|  |  |  |  |  | saw | /tmp/in/input.doc |  |  |  |
|  | IMPORTANT: SUBMIT QUOTATION IN SEALED PACKAGE WITH QUOTATION NUMBER, CLOSING DATE AND BUYER’S NAME MARKED CLEARLY ON THE OUTSIDE TO: |  |  |  |  |  |  |  |  |
|  | COUNTY OF FRESNO, Purchasing 4525 EAST HAMILTON AVENUE, 2ND FLOOR FRESNO, CA 93702-4599 |  |  |  |  |  |  |  |  |
|  | Closing date of bid will be at 2:00 p.m., on June 7, 2011. QUOTES WILL BE CONSIDERED LATE WHEN THE OFFICIAL PURCHASING TIME CLOCK READS 2:00 P.M. |  |  |  |  |  |  |  |  |
|  | Quotes will be opened and publicly read at that time. All quotation information will be available for review after contract award. |  |  |  |  |  |  |  |  |
|  | Clarifications of specifications are to be directed to: Craig Nickel, phone (559) 600-7110, FAX (559) 456-7831. |  |  |  |  |  |  |  |  |
|  | NOTE THE following and attached ADDITIONS, DELETIONS AND/or CHANGES TO THE REQUIREMENTS OF REQUEST FOR QUOTATION NUMBER: AND INCLUDE THEM IN YOUR RESPONSE. PLEASE SIGN AND RETURN THIS ADDENDUM WITH YOUR QUOTATION. - The Close Date has been changed to June 7, 2011 at 2:00 p.m. - Engine Information Certification Label Sheet (attached) shall be completed by the contractor. - Add two (2) rear flail mowers to be installed by County to this RFQ. Insert the attached specifications and Quotation Schedule. - Delete Page 46; QUOTATION SCHEDULE REPURCHASE AGREEMENT. Insert in its place: QUOTATION SCHEDULE REPURCHASE AGREEMENT (revised). |  |  |  |  |  |  |  |  |

| ACKNOWLEDGMENT OF ADDENDUM NUMBER TWO TO RFQ |  |  |
| --- | --- | --- |
| COMPANY NAME: |  |  |
| (PRINT) |  |  |
| SIGNATURE: |  |  |
| NAME & TITLE: |  |  |
| (PRINT) |  |  |

Reference Page 12; EQUIPMENT MANUALS

Delete: A.  One (1) operator’s manual for each unit.

Insert in its place: A.  Three (3) operator’s manuals for each unit.

Reference Page 22; ENGINE

Four (4) valves per cylinder design.

The intent of specifying the four (4) valves per cylinder design is to satisfy engine emission (engineering) requirements. Other designs meeting this criteria that are submitted with the bid package, will be evaluated.

Insert: Vendor shall provide additional factory supplied engine label, per CARB requirements. One shall be placed on the equipment; one is to be kept in the Fleet Services Office, as the labels placed on the equipment become obscured over time.

Insert: The Tractor shall, at a minimum, meet Tier III emission requirements.

Reference Page 26; WARRANTY

Delete: 24 months or 2,000 hours, whichever comes first.

Insert in its place: 36 months or 2,100 hour, whichever comes first.

Reference Page 29; A Polycarbonate lexan window shall be installed to provide safety protection for the operator.

Delete: It can be installed in place of, or over the glass, or attached to a frame on the right side of the cab windows.

Insert in its place: It shall be frame mounted outside of glass to protect window and operator.

Reference Page 37;

Delete: Left rear wheel of the Tractor shall be mounted with a 1,700 lb. inner wheel weight, and also an 850 lb. outer wheel weight.

Insert in its place: Contractor shall apply appropriate weight for this application.

Delete Page 46, QUOTATION SCHEDULE REPURCHASE AGREEMENT.

Insert in its place (attached):

QUOTATION SCHEDULE REPURCHASE AGREEMENT (revised)

ENGINE INFORMATION CERTIFICATION LABEL

Please provide a copy/facsimile of the manufacturer’s factory engine identification label which is required to be attached to the equipment engine providing information including but not limited to the following:

| Engine Serial Number: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Manufactured By: |  |  |  |  |
| Date of Manufacture: |  |  |  |  |
| Ref. No.: |  |  |  |  |
| Engine Model: |  |  |  |  |
| Engine Family: |  |  |  |  |
| Idle Speed: |  |  |  |  |
| Advertised HP: |  |  |  |  |
| Maximum HP: |  |  |  |  |
| Firing Order: |  |  |  |  |
| Timing: |  |  |  |  |
| Valve Lash Cold: | Int. |  | Exh |  |
| Displacement (Liters): |  |  |  |  |
| E.C.S EGR.PTOX.SCRC NOX |  |  |  |  |

Important Engine Information: This engine conforms to US EPA and California regulations applicable to 20__ Model Year new heavy duty engines. This engine is certified to operate on diesel fuel.

SPECIFICATIONS 90" HD CENTERED REAR PTO FLAIL

| SPECIFICATIONS REQUIRED | COMPLY (Yes/No) | DEVIATIONS/EXCEPTIONS/COMMENTS |
| --- | --- | --- |
| General |  |  |
| It is the purpose of the following specifications to describe a self-propelled, mechanically driven, rear mounted, flail mower, which shall mow behind the tractor's rear tires. The unit shall be of the manufacturer's current production model, meeting or exceeding the terms of these specifications. Unit(s) shall be the manufacturer's most heavy-duty model available. The manufacturer shall furnish parts and operation manuals for the unit(s) bid. The manufacturer shall also guarantee equipment against defects in workmanship and materials for a period of (1) year. |  |  |
| SAFETY SPECIFICATIONS |  |  |
| ●Unit shall meet or exceed the following industry standards: SAE J1001, J115, J208, J715, J898, J909, J920, J1150, J1500. ANSI/ASAE S441, S318.7, S217.10, S278.6, EP363. ASTM A370. A compliance report shall be available upon request. |  |  |
| CUTTING HEAD |  |  |
| ●Cutting width shall be 90" of actual cut. |  |  |
| ●Cutting assembly shall be flail type, and mechanically powered. |  |  |
| ●Cutting shaft shall have a minimum of (96) all purpose, self cleaning, reversible knives, or (48) smooth cut cutting knives. It shall not be necessary to change the cutting shaft to interchange knives. All purpose knives are 6 gauge thickness, 1-3/4" wide x 4-5/8" length. Smooth cut knives are 1/4" thickness, 2-1/2" wide x 3-7/8" length. |  |  |
| ●Cutting shaft shall be a minimum of 6" in diameter, with 2-3/16" diameter stub shafts and bearings. Cutting shaft bearings will be of the grip tight double shielded type. |  |  |
| ●Cutting heads shall have 6-5/8" diameter rollers, 1-3/8" grip tight shielded bearings, and replaceable 1-3/8" stub shafts with external bearing hub caps and cap retainers. |  |  |
| ●Cutting head height shall be infinitely adjustable from -1/2" to +6". Height adjustment shall be accomplished thru an externally mounted, threaded, screw type adjuster. It shall not be necessary to remove the roller mounting brackets to adjust cutting height. |  |  |
| ●Cutting head weight shall be a minimum of 1440 lbs. |  |  |
| ●Mechanical motor assemblies shall have automatic belt self-tensioners (specifically excluding back bend type belt tensioner.) Drive belt system shall have a minimum rating of 80hp. |  |  |
| ●Cutting head shall have a minimum of 1/2" thick solid steel sidewalls. |  |  |
| ●Cutting bonnet thickness shall be a minimum of 3/16". |  |  |
| ●Rear flail will be equipped with a category II, 3 point hitch. The flail will be centered from the mounting point. |  |  |
| ●Rear flail shall be equipped with a safety discharge flap and front deflector shield. |  |  |
| ●Gearbox shall be rated @ 90hp constant, 135hp intermittent, and shall be reversible. |  |  |
| ●Cutting shaft rpm shall be a minimum of 1750 (tip speed of 9050 fpm.) |  |  |
| ●Cutting head shall have removable skid shoes. |  |  |

| Company Name |  |
| --- | --- |

QUOTATION SCHEDULE

| 90” HD Centered Rear PTO Flail, County shall install. . |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | 2 | EA | $ |
| Brand: |  |  |  |  |
| Model: |  |  |  |  |
| Other: List any other applicable charges |  |  |  |  |
| Sales Tax (8.975%) |  |  |  | $ |
| TOTAL |  |  |  | $ |